Comparison to standard GMH cams

|  |  | VN Series 1 V6 Engine | Late VN-VP-VR-early VS Series Two V6 Engine | Late VS-VT-VX-VY EcoTech V6 Engine | Frankenstein Cam |
| --- | --- | --- | --- | --- | --- |
| Advertised | Advertised Valve Timing at height | 0.006” | 0.008” | 0.006” | 0.006” |
|  | Inlet Opens | 22.4º BTDC | 9.8º BTDC | 6.7º BTDC | 35.11º BTDC |
|  | Inlet Closes | 66.9º ABDC | 52.1º ABDC | 57.0º ABDC | 74.35º ABDC |
|  | Exhaust Opens | 70.7º BBDC | 56.8º BBDC | 65.3º BBDC | 73.57º BBDC |
|  | Exhaust Closes | 26.2º ATDC | 14.5º ATDC | 15.1º ATDC | 38.66º ATDC |
|  | Inlet Valve Duration | 269.3º | 241.9º | 243.7º | 289.47º |
|  | Exhaust Valve Duration | 276.9º | 251.3º | 260.4º | 292.23º |
|  | Inlet Valve Lift with 1.6 to 1 ratio rockers | 0.4376” | 0.4066” | 0.4099” | 0.56132” |
|  | Exhaust Valve Lift with 1.6 to 1 ratio rockers | 0.4373” | 0.4083” | 0.4112” | 0.56044” |
|  | Inlet Valve Lift at TDC | 0.0336” | 0.0256” | 0.0208” | 0.0760” |
| @ 0.050” | Inlet Opens | -15.3º BTDC (15.3º ATDC) | -20.9º BTDC (20.9º ATDC) | -24.4º BTDC (24.4º ATDC) |  |
|  | Inlet Closes | 29.2º ABDC | 21.4º ABDC | 25.8º ABDC |  |
|  | Exhaust Opens | 30.4º BBDC | 26.1º BBDC | 31.6º BBDC |  |
|  | Exhaust Closes | -14.2º degrees ATDC (14.2º BTDC) | -16.2º degrees ATDC (16.2º BTDC) | -18.7º degrees ATDC (18.7º BTDC) |  |
|  | Inlet Valve Duration | 193.9º | 180.5º | 181.4º |  |
|  | Exhaust Valve Duration | 196.2º | 189.9º | 192.9º |  |
|  | Inlet Valve Lift with 1.6 to 1 ratio rockers | 0.4376” | 0.4066” | 0.4099” |  |
|  | Exhaust Valve Lift with 1.6 to 1 ratio rockers | 0.4373” | 0.4083” | 0.4112” |  |
|  | Ramp Rate | 2.13 | 2.27 | 2.22 |  |
|  | Valve Overlap Seat to Seat | 48.6º | 24.3º | 21.8º |  |
|  | Valve Overlap at 0.050” | -29.5º | -37.1º | -43.1º |  |
|  | Inlet Lobe Centreline | 112.2º | 111.1º | 115.2º |  |
|  | Exhaust Lobe Centreline | 112.3º | 111.2º | 115.1º |  |
|  | Lobe Separation Angle - | 112.3º | 111.2º | 115.1º |  |

Comparison to Camtech cams for Holden V6 VN Series 2 - VR 1991-1993.

| Part Nº | Description | Duration |  |  |  | Valve Lift with 1.6 Rocker |  | Lobe Sep Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Advertised |  | @.050” |  |  |  |  |
|  |  | IN | EX | IN | EX | IN | EX |  |
| CT2774 49902B-113 | Smooth idle, strong torque and enhances throttle response. Suits both petrol & LPG. RPM RANGE 1500-5300 | 253˚ | 253˚ | 196˚ | 196˚ | 0.427" | 0.427" | 113˚ |
| CT2774 489 | Performance street use. Strong mid range. Suits both petrol & LPG. RPM RANGE 1800-5800 | 270˚ | 276˚ | 208˚ | 213˚ | 0.468" | 0.468" | 112˚ |
| CT2774 49937-110 | Performance street use. Very strong-mid range, torque & throttle response. Min 9.5:1 comp ratio & 3.5:1+ diff ratio. RPM RANGE 2500-6200 | 278˚ | 283˚ | 217˚ | 221˚ | 0.530" | 0.530" | 110˚ |
| CT2774 49928-110 | Performance street use. Very strong-mid range, torque & throttle response. Min 9.5:1 comp ratio & 3.5:1+ diff ratio. RPM RANGE 2800-6500 | 278˚ | 286˚ | 222˚ | 230˚ | 0.542" | 0.563" | 110˚ |
| CT2774 49903 | Performance street use. Great for weekend cruise car and street strip meetings. Very strong mid range.Min comp ratio 10:1. Best with 3.5:1+ diff ratio. RPM RANGE 3000-6800 | 286˚ | 288˚ | 230˚ | 233˚ | 0.561" | 0.561" | 110˚ |
| Frankenstein Cam | Awesome for punting yellow utes around. Harv needs a turn. | 289.47º | 292.23º | 230.18 | 230.36º | 0.5613” | 0.56044” | 110º |
| CT2774 49901B/49933B-110 | Street / strip. Rough idle. Strong mid range and top end. Min comp ratio 10:1. Best with 3.7:1+ diff ratio. RPM RANGE 3500-7000 | 290˚ | 300˚ | 237˚ | 244˚ | 0.570" | 0.576" | 110˚ |